Assignment No 5 (Group Activity)

Code 1:

#include <DHT.h>

#define DHTPIN 4

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(115200);

dht.begin();

Serial.println("Temperature,Humidity");

}

void loop() {

float temperature = dht.readTemperature();

float humidity = dht.readHumidity();

if (isnan(temperature) || isnan(humidity)) {

Serial.println("Sensor read error");

delay(2000);

return;

}

Serial.print(temperature);

Serial.print(",");

Serial.println(humidity);

delay(5000);

}

Code 2: collect_data.py

import serial

import csv

import time

from datetime import datetime

port = "/dev/cu.SLAB_USBtoUART"

baud_rate = 115200

total_readings = 100  # Increased for a better analysis pool

try:

ser = serial.Serial(port, baud_rate, timeout=10)

time.sleep(2)  # Allow link state to settle

except serial.SerialException as e:

print(f" Serial Port Error: {e}")

print(" Unplug/replug USB or verify the Arduino IDE Serial Monitor is shut.")

exit()

with open("industrial_sensor_data.csv", "w", newline="") as file:

writer = csv.writer(file)

writer.writerow(["Timestamp", "Index", "Temperature", "Humidity"])

count = 0

print(" Ingesting hardware data... Press Ctrl+C to terminate early.")

while count < total_readings:

line = ser.readline().decode("utf-8", errors="ignore").strip()

values = line.split(",")

if len(values) == 2:

try:

temp = float(values[0])

hum = float(values[1])

timestamp = datetime.now().strftime("%Y-%m-%d %H:%M:%S")

count += 1

writer.writerow([timestamp, count, temp, hum])

file.flush()  # Commit data directly to the disk matrix

print(f" Record [{count}/{total_readings}] Ingested -> Temp: {temp}°C | Hum: {hum}%")

except ValueError:

continue

ser.close()

print(" Dataset successfully aggregated inside industrial_sensor_data.csv")

Code 3: analyze_data.py

import pandas as pd

import matplotlib.pyplot as plt

# 1. Load the generated enterprise log

try:

df = pd.read_csv("industrial_sensor_data.csv")

print(" Dataset loaded successfully.")

print(df.describe())  # Prints key mathematical summaries directly to console

except FileNotFoundError:

print(" Dataset not found! Run the collection script first.")

exit()

# 2. Configure visualization canvases

plt.figure(figsize=(12, 6))

# Plot Temperature Metrics

plt.subplot(2, 1, 1)

plt.plot(df['Index'], df['Temperature'], color='red', marker='o', linestyle='-', linewidth=1.5)

plt.title("Industrial Environmental Profile Analysis")

plt.ylabel("Temperature (°C)")

plt.grid(True, linestyle='--', alpha=0.6)

# Plot Humidity Metrics

plt.subplot(2, 1, 2)

plt.plot(df['Index'], df['Humidity'], color='blue', marker='x', linestyle='-', linewidth=1.5)

plt.xlabel("Reading Reference Target (Index)")

plt.ylabel("Relative Humidity (%)")

plt.grid(True, linestyle='--', alpha=0.6)

plt.tight_layout()

# 3. Export assets for the group laboratory report

plt.savefig("industrial_analysis_trends.png", dpi=300)

print(" Analytic visualization exported as 'industrial_analysis_trends.png'")

plt.show()

Output: